I.A.R.

Investigation  Analysis Solutions

Contracting Authority :  „Balts un Melns” LTD

Design stage: Technical project

Geotechnical investigation

work report

For Valga – Valka twin city center development

Chairperson of the Board:		B. Arāja

Geotechnical engineer:		G. Robalts

Field geologist:				G. Robalts

Rīga 2017

LTD „I.A.R.”, Grobiņa Municipality., Grobiņa rural theritory., “Robalti”, LV-3430, Mob. tel. 29466195

LTD „I.A.R.” Geotechnical engineer Gints Robalts Latvian Association of Civil Engineers (LACE) bulding trade certificate 20-6929

Grobiņa Municipality., Grobiņa rural territory., “Robalti”, tel. 29466195, e-mail robalts@inbox.lv

Geotechnical investigation report

Valga – Valka twin city center development, Rīgas, Raiņa, and Latgales Street, Valka Municipality,Valka city, Latvia Raja, Tartu, and Sōpruse Street, Valga city, Estonia

(the name and the address of the building)

„ Balts un Melns” LTD

(Contracting Authority)

Geotechnical investigation work task from October 1, 2017

(inspection task, its issue date)

1. General information regarding the investigation object

| 1.1. | Building kind | Territory development |
| --- | --- | --- |
| 1.2. | Investigation territory | Rīgas, Raiņa, and Latgales Street, Valka Municipality,Valka city, Latvia Raja, Tartu, and Sōpruse Street, Valga city, Estonia |

2. General information regarding the investigation methids and amounts

| 2.1. | Investigation kinds | Threading 15 pieces. |
| --- | --- | --- |
|  |  | Static probing(CPT ) 1 piece |
|  |  | Dynamic probing (DPL) 4 pieces |
| 2.2. | Development amount | 15 |
| 2.3. | Laboratory tests | Granulometric composition 16 pieces. (LVS CEN ISO/TS 17892-4:2005) Atterberg limit 2 pieces. Organic substance composition 6 pieces Ground agressiveness on concrete 1 piece, and on iron constructions 1 piece |

3. The present situation

| 3.1. | City territory |
| --- | --- |
| The investigation territory is located at: Rīgas, Raiņa, and Latgales Street, Valka Municipality, Valka city, Latvia; Raja, Tartu, and Sōpruse Street, Valga city, Estonia image 1 The territory is geomorphologically included in Tālava lowland, in the Northern part of Seda plain. The relief of the investigated theritorry is quite even, but with a slight rise from Estonian-Latvian side towards Valka city center. The absolute altitude marks start from 45,5 – 51,0 m above sea level. The geological composition up to the depth of 15,0 m in the investigation points is made of quaternary sediments: 1) technogenic sediments – asphalt, heaped up grounds; 2) the Baltic ice lake limnoglacial sediments – thin, dusty sand and sandy loam; 3) river and bog sediments – the sludge is sandy and with addition of and organics. See a more detailed geotechnical section in the drilling journals and geotechnical sections. |  |

4. Summary

| 4.1. | Geotechnical description of the field |
| --- | --- |
| During the geotechnical investigation (drilling, static probing, and laboatory test results) it has been concluded that the engineering geological section up to the depth of 15,50 m is composed of the following grounds: 1. (siOr) Soil (ĢTE-2), sandy, starting from very unconsolidated up to medium dense, humid, has been found at the upper part of the section 0,10 - 0,30 m thick (in drillings PA12, PA13, IP1, IP3-IP5). 2. (FSaMg) Technogenous ground - asphalt (ĢTE-A) found at the beginning of the section, 0,10 – 0,20 m thick (in drillings PA1 – PA3, PA6-PA9). 3. (FSaMg) Embanked ground– fragments with fine sand (ĢTE-1gr) compacted up to a very dense condition, dry, located under the asphalt layer in depth of 0,20 – 0,60 m (in drillings PA1-PA3, PA6-PA9, and IP6). 3. (FSaMg) Embanked ground – fine sand (ĢTE-1s) dense, dry, which has been found in drillings PA1-PA3, PA7-PA9, and IP5. The thickness of ground is 0,30 -2,10 m. 4. (FSaMg) Embanked ground – medium sand (ĢTE-1m) is medium dense, dry, but wet along with the ground level, found in the drilling PA1 with thickness of 0,80 m. 5. (FSaMg) Embanked ground – fine sand with soil and debris (ĢTE-1sbr) compressed, dry, found in the drilling IP1. with thickness of 1,50 m. 6. (siSa) Dusty sand (ĢTE -63) medium dense, dry and saturated with water, found in the drillings PA8 and IP5. A dusty sand layer has been found in thickness of 0,40 m (PA8) and 1,1 m (IP5). The layer was not perforated. 7. (FSaP) Fine sand (ĢTE-75) very loose, water-saturated, foud in the drilling IP2 with thickness of 0,60 m. 6. (FSaP) Fine sand (ĢTE-74) loose, with organics, wet and water-saturated, found in the drillings PA12, PA13, IP1-IP3, IUP5 with the thickness of 0,2 – 3,8 m at different heights of the section. 6. (FSaP)Fine sand (ĢTE - 73) average dense, dry, and water-saturated, found in the drillings PA7-PA9, PA12, PA13, IP2-IP6 at the lower part of the section with thickness of 0,3 – 3,9 m. The layer was perforatd only in the drillings PA8, PA13 and IP5. 6. (FSaP) Fine sand (ĢTE – 72) dense, water-saturated, found in the drillings PA2, PA13 and IP1 at the lower part of the section with thickness of 0,1 m (PA13) 1,4 m (PA2) and 9,6 m (IP1), the layer was not perforated. 7. (OH organic clay with sandy interlayers) Sludge is sandy (ĢTE – 9mp, 9p) softly plastic, fluent, water-saturated, sandy, found in the drilling IP2 in the middle part of the section with thickness of 1,9 m. 5. (ClL) Loamy soil (ĢTE – 14s), tenacious, wet; found in the drilling IP4 at the upper and middle parts of the section with thickness of 4,10 m. 6. (ClI) Sandy loam (ĢTE – 15s), tenacious, wet; found in the drilling PA13 at the upper part of the section with thickness of 0,20 m. See a more detailed geotechnical section in the drilling journals and in the geotechnical section 1-1’, as well as the ground physically mechanical property table. 7. Groundwater level on 29.10.2017. was found for PA1, PA2, PA8, PA12, PA13, IP1-IP4 in depth of 0,10 - 3,50 m of the ground surface. The groundwater level monitoring was not xecuted at the investigation territory. 8. The ground corrosion activity towrds steel is high, and the average density of the ground kcathode current A/m2 is 0,279. See Annex 5. The information gained within the work frame reflects the situation at the specific place an it has been assumed that it is similar in other places as well, where the tests have not been executed. However, it is possible that at some places the situation or conditions may differ from the assumed ones, and that has not been found. |  |
| 4.2. | Conclusions and suggestions 1. The geotechnical conditions in the investigated territory are inconsistent. They are good for a bridge building, because the piles will be based on firm grounds. The foundation must be executed following the ground physically mechanical properties (Annex 1). 2. At the upper and the middle part of the investigated theritory geotechnical section At the upper and the middle part of the investigated territory geotechnical section grounds with weak load carrying capacity have been found ĢTE - 75, ĢTE – 74, ĢTE – 84, ĢTE – 9mp, 9p, loose and very loose fine sand and sandy sludge layers (E= 3.11 Mpa up to 17.15 Mpa). It is suggested not to use these grunds for the foundation of the newly erected building, otherwise an uneven building compaction and basement deformation may take place. 3. In a case of the pile foundation usage it is suggested to use dense, fine sand (ĢTE-72) (E = 31.68 MPa). 4. The groundwater level was found on 29.10.2017. in PA1, PA2, PA8, PA12, PA13, IP1-IP4 starting from the depth of 0,10 - 3,50 m from the ground surface, but depending on the season and the amount of precipitation it can change within the borders of ±0,50 m. 5. Correspondingly to LBN 003 - 15 „Construction climatology” the sandy ground freezing depth in the territory is 1,38 m once in 10 years. |

„I.A.R.” LTD geotechnical engineer/ geologist: Gints Robalts

The geotechnical investigation was executed on October 29. 2017.

| Text annexes |  |  |
| --- | --- | --- |
| 1. | Physically mechanical indicators of the ground | 1 page |
| 2. | Static probing data (graphic and probilng data) | 1 page |
| 3. | Ground testing report | 2 pages |
| 4 | Ground chemical analyses | 1 page |
| 5. | Teotechnical investigation certificate (Gints Robalts) | 1 page |
| Graphical annexes |  |  |
| 1. | Geotechnical development location plan M 1 : 500 | 2 pages |
| 2. | Drilling descriptions | 7 pages |